Biomethane Operator Assistant

Technical Capabilities Overview

Version 5  •  May 2026  •  Farm-Scale Biogas AI Assistant

Abstract

The Biomethane Operator Assistant is a local AI assistant system designed to support farm-scale anaerobic digestion operations. It connects a locally running language model to a structured toolbox of process calculations, physical simulation, and state estimation — all running on a single machine with no data leaving the farm.

The system is built on honest principles: every tool clearly distinguishes between what is calculated from first principles, what is estimated from a physical model with known uncertainty, and what is directly measured. The AI assistant acts as a knowledgeable first responder — available 24 hours a day to interpret sensor readings, flag stability risks, and support operational decisions. It is not a replacement for a process engineer.

| DESIGN PRINCIPLE | This system runs entirely on-premises. No sensor data, operational records, or AI queries leave the farm network. All uncertainty in model outputs is stated explicitly in every tool response. |
| --- | --- |

Note

- Data sovereignty is a relative term.  Through the use of local LLM, no data leave the farm network to crowd LLM, however, the data communicated through WAN/LAN is exposed.  Encrypted communication is necessary.

- The system (Proof Of Concept) is being developed, using M4 Mac mini with 24 G RAM.  Computer hardware with better CPU and RAM can house more sophisticated local LLM.   This will improve logical reasoning capabilities and reduces the chance of AI hallucination.

1.  System Architecture

## 1.1  The AI Client and MCP Server

## The system uses two components working together:

- AI Client — A locally running language model (e.g. Gemma 4 4B or equivalent). The AI interprets operator questions, retrieves relevant knowledge from the RAG document store, calls the appropriate MCP tools, and explains results in plain language.

- MCP Server — bio_methane_operations_mcp_server_v5.py is a structured toolbox of callable functions covering plant monitoring, feedstock chemistry, physical simulation, and state estimation. Runs as a stdio or HTTP process on the same machine.

Transport: The MCP server runs over stdio by default — it is launched as a subprocess by biomethane_chat.py which communicates via stdin/stdout JSON-RPC messages. No network ports are opened for MCP unless the --http flag is passed explicitly. This means the chat UI and MCP server share a single process tree and no external service needs to be started or managed separately.

When an operator asks “Is my digester safe right now?” the AI calls the relevant MCP tools, reads the results, and responds with an explanation grounded in real thresholds and physics — not general advice.

## 1.2  Two-Tier Storage Architecture

## Version 5 introduces a two-tier storage design that separates clean operational data from legacy seeding:

| Store | File | Role |
| --- | --- | --- |
| StateBuffer | data/plant_state.sqlite | Primary store for all live and replayed sensor readings. Every reading passes through CUSUM anomaly detection before persisting. get_plant_state() reads the latest clean row from StateBuffer. |
| biomethane.db | data/biomethane.db | Legacy key-value store retained for default seeding and backward compatibility with v3 tests. |

## 1.3  RAG Knowledge Layer

## Four curated documents are embedded in a ChromaDB vector store and retrieved by the AI when answering questions. These are purpose-written for a small parameter model — dense, lookup-style, and explicit about uncertainty:

- AD4 Model Card — state variables, parameters, operating ranges, temperature correction tables

- Bernard (2001) Equations — 10 discrete chunks, one equation per chunk, for precise retrieval

- API Reference Sheet — every tool input and output field defined in plain language

- Failure Mode Lookup — 11 diagnostic tables covering S2, X2, methane, temperature, EnKF outputs, and tool selection guidance

## 1.4  Integration Scripts

## Five helper scripts complete the data pipeline from raw SCADA export to live AI-accessible state:

scada_mapper.py — Think of scada_mapper.py as a universal translator for SCADA files. Every biogas plant’s SCADA system uses different column names — Siemens calls temperature TIC_101, Rockwell calls it TEMPERATURE_PV, and a generic CSV might just say DigesterTemp. This script auto-detects the format (Siemens, Rockwell, Mitsubishi, or generic), then maps every column to a standard internal name (digester_temp_c, biogas_flow_nm3h, ch4_pct, etc.) that the rest of the system understands.

Depends on: pandas, re (stdlib). Used by ingest_daemon, replay_dataset, seed_plant_state, and the MCP server’s ingest_scada_file() tool.

StateBuffer.py — StateBuffer.py is the system’s short-term memory for sensor data. It is a SQLite-backed rolling buffer that holds 48 hours of clean sensor readings. Every incoming measurement passes through a CUSUM (Cumulative Sum) control chart — a statistical filter that catches sensor glitches, stuck probes, and outlier readings before they pollute the plant state. Think of it as a quality gate: noisy data goes in, clean trends come out. The MCP server reads its plant state from this buffer, not from raw SCADA files.

Depends on: sqlite3, pandas. Used by the MCP server, replay_dataset, seed_plant_state, and ingest_daemon.

ingest_daemon.py — ingest_daemon.py is the automatic file watcher. It monitors a directory (default: scada_exports/) for new CSV files from your SCADA system, runs each file through scada_mapper → StateBuffer automatically, and logs what it ingested so it never processes the same file twice. Set it up as a cron job or systemd service and forget about it — new sensor data flows into the system without manual intervention.

Depends on: scada_mapper, StateBuffer. Typically run via cron or as a background service.

replay_dataset.py — replay_dataset.py is the time machine. It takes a historical CSV dataset (like the ri_flex.csv sample data) and replays it day-by-day through the full pipeline: scada_mapper reads the columns, StateBuffer applies CUSUM filtering, the MCP server updates plant state, and EnKF advances its state estimate. This is how you validate that the system would have caught last year’s souring event three days early, or test how different CUSUM parameters filter sensor noise. The output is a detailed JSON report with per-day results and summary statistics.

Depends on: scada_mapper, StateBuffer, MCP server (launched as subprocess). Expects a CSV file at sample_data/ri_flex.csv by default.

seed_plant_state.py — seed_plant_state.py bootstraps the system on first setup. It reads a CSV dataset (typically ri_flex.csv or your plant’s historical records), runs it through scada_mapper and StateBuffer, computes baseline statistics (mean biogas production, temperature range, estimated HRT, organic loading rate), and writes the initial plant state into the MCP database. Without this step, the system starts with factory defaults and needs a week of live data to converge to realistic values.

Depends on: scada_mapper, StateBuffer, MCP server DB (data/biomethane.db). Supports --period (all/last_90/last_30/last_7) and --dry-run.

## 1.5  CLI Tool with integrated Calibration

The CLI Tool (bio_cli.py) runs locally on the same hardware that houses the MCP Server. It provides two categories of operations:

- Basic Operations: plant state, alerts, blending, KPIs, references (same as MCP Server tools)

- Calibration: physics-based and SCADA-fitting functions INTEGRATED into the CLI Tool

biomethane_calibration.py — biomethane_calibration.py is the physics engine behind the CLI’s calibration tools. It contains the Buswell equation for theoretical methane potential, C/N ratio calculations, energy conversion factors, and the AD4 parameter fitting routines (analytical k6 estimation from historical production data). When you run bio_cli.py calibrate buswell --c 6 --h 12 --o 6, this is the script doing the actual work. It is a library, not a standalone tool — bio_cli.py imports and wraps every function.

Depends on: ad4_simulator, pandas, numpy. Not called directly by the MCP server; all calibration goes through bio_cli.py.

Key use cases:

- Runs on local terminal when you have direct access to the server machine

- Emergency operations when AI services are unavailable

- Calibration without AI (offline)

- Automated scripts and cron jobs

Note

CLI Tool with Calibration is NOT exposed through MCP Protocol. It executes directly as a Python script on the local machine.

## 1.6  AI Interface Scripts

Two scripts bridge the gap between the operator and the MCP server, providing structured dispatch and a web chat interface.

biomethane_router.py — biomethane_router.py is the intelligent dispatcher. When an operator types a question like “Is my digester safe?” or “What happens if I double the feeding rate?”, this script classifies the intent (STATUS, SOURING, SCENARIO, PRODUCTION, REFERENCE) by matching keywords in the message, then runs the appropriate chain of MCP tool calls automatically. It saves the AI from having to figure out which tools to call and in what order — the routing logic is deterministic, tested, and consistent.

Depends on: MCP client (mcp Python package). Used by biomethane_chat.py as the backend for structured queries.

biomethane_chat.py — biomethane_chat.py is the web-based chat interface. Built with Gradio, it provides a browser UI where operators type questions and get answers from the AI assistant. It launches the MCP server as a subprocess, connects to a local LLM (e.g. Gemma 4 running via llama-server), and manages the conversation turn by turn. Every response includes a tool trace panel showing exactly which MCP tools were called and what they returned — full transparency. Run it with python src/biomethane_chat.py and open http://localhost:7860 in your browser.

Depends on: Gradio, MCP server (bio_methane_operations_mcp_server_v5.py), and a running LLM server (e.g. llama-server with Gemma 4). Optionally uses biomethane_router.py for structured query dispatch.

## 1.7  Site Configuration

The file site_config.json (auto-discovered at PROJECT_ROOT/site_config.json or src/site_config.json) is the central configuration file read by the MCP server, StateBuffer, CLI tool, SCADA mapper, ingest daemon, and router. A single JSON object controls all runtime parameters:

Digester geometry — digester_volume_m3, hrt_days, feed_kg_vs_day, s1_in_g_per_l. Used by the EnKF for auto-initialisation at startup and by the router for simulation inputs.

Temperature profile — the lab_scale boolean toggles between lab-scale (20–30 °C) and industrial (35–40 °C) alert thresholds.

Alert thresholds — two profiles (lab_scale and industrial) each define low/medium/high or min/max boundaries for every monitored sensor (temperature, VFA, NH₄, H₂S, pH, CH₄ purity, O₂). Any key not present falls back to hardcoded defaults.

CUSUM filter parameters — per-sensor K (allowable slack) and H (decision interval) values that control how aggressively StateBuffer rejects spikes.

Daily assessment thresholds — souring_probability_watch (0.10), souring_probability_action (0.30), s2_rising_days (2), and run_hour_utc (6) control the background assessment thread’s advisory and escalation logic.

SCADA tag mapping — an optional tag_mapping object that lets scada_mapper skip vendor auto-detection and use a static column-name map instead.

Ingest daemon settings — watch_dir, file_pattern, lookback_minutes, poll_subfolders, and biogas_scale_factor configure the automatic file watcher.

The CLI flag --site PATH overrides the auto-discovery path. See docs/api/site_config.md for the full schema reference.

2.  Tool Layers and Honest Capabilities

## 2.1  Layer 1 — Plant State Monitoring

## Reads and writes the live StateBuffer of plant sensor values. This layer is purely data management — no calculations are performed here.

| Tool | Purpose | Data source |
| --- | --- | --- |
| get_plant_state | Read all current sensor values | StateBuffer (latest clean row) |
| update_plant_state | Write new sensor readings; routed through StateBuffer CUSUM | StateBuffer → plant_state.sqlite |
| check_alerts | Evaluate thresholds, return active alerts | Literature thresholds (VDI 4630) |
| get_alert_history | Return recent alert log (configurable limit) | In-memory alert deque |
| get_vfa_alkalinity_ratio | Calculate FOS/TAC ratio and assess acidification risk (optimal 0.1–0.3) | vfa_mmol_l + alkalinity_mg_caco3_l from plant state |
| get_kpi_summary | Revenue, energy, CO₂ avoided estimates (daily / weekly / monthly) | Derived from sensor values |
| get_state_buffer_status | StateBuffer health: row count, latest clean readings, CUSUM accumulator state, sensors near threshold | StateBuffer diagnostics |
| ingest_scada_file | Ingest a SCADA export (CSV or Excel) via the full scada_mapper → StateBuffer pipeline. Called directly by the AI when the operator provides a data file. | File path; auto-detects vendor |

## 2.2  Layer 2 — Feedstock Chemistry (First Principles)

## Physics-based calculations requiring no measured data from your plant. Results are deterministic given feedstock composition.

- buswell_bmp() — theoretical maximum methane yield from molecular formula (Buswell & Mueller 1952)

- buswell_bmp_by_class() — BMP by substrate type (lipid, carbohydrate, protein)

- cn_ratio_from_composition() — carbon-to-nitrogen ratio from elemental analysis

- olr_from_recipe() — organic loading rate from a feedstock recipe and digester volume

- biodegradability_coefficient() — compares measured BMP to theoretical Buswell maximum

- calculate_energy_conversion_factor() — LHV-based energy output from CH₄ fraction

- blend_feedstocks() — NEW in v5: calculates a multi-stream feedstock blend to a target OLR, reports C/N ratio, estimated CH₄ yield, daily biomethane output (Nm³ and MWh), and inhibition warnings for high-nitrogen or high-H₂S substrates

- list_feedstocks() — returns the built-in feedstock database with BMP, DM, VS, and C/N values

- get_operational_reference() — NEW in v5: plain-language lookup table for process topics (fos_tac, temperature, OLR, C/N ratio, etc.)

| HONEST LIMIT | Layer 2 computes what is theoretically possible from feedstock chemistry. Actual plant performance (50–95% of theoretical) depends on operating conditions not captured in Buswell equations. Use Layer 2 for feedstock planning and recipe optimisation, not production forecasting. |
| --- | --- |

## 2.3  Layer 3 — Physical Simulation (AD4 ODE Model)

## Three tools wrap the AD4 simulator — a 4-state ordinary differential equation model of anaerobic digestion (Bernard et al. 2001). The model tracks organic substrate (S1), volatile fatty acids (S2), acidogen biomass (X1), and methanogen biomass (X2).

## Temperature correction: All three tools apply an Arrhenius correction to the methanogen growth rate (mu2_max) using the live digester temperature from plant_state. A 3°C cooling reduces the safe operating envelope (D_crit) by approximately 10%. This is applied automatically.

| Tool | Question it answers | Key output |
| --- | --- | --- |
| ad4_simulate | What steady state should I expect at my current HRT and loading? | S2, X2, methane rate, souring/washout flags |
| ad4_critical_dilution_rate | How close am I to the washout threshold at today’s temperature? | D_crit, safety margin %, SAFE / CAUTION / WARNING |
| ad4_perturbation_test | What happens if I receive a high-COD delivery for 10 days? | Peak S2, min X2, recovery outcome |

| HONEST LIMIT | Layer 3 provides qualitative stability analysis, not quantitative production forecasting. Kinetic parameters (Ki2, mu2_max) are Benyahia (2012) defaults derived from lab-scale reactors — not calibrated to your specific plant population. Use these tools for what-if reasoning and washout risk assessment. For methane yield prediction, use Layer 2 (Buswell + OLR). |
| --- | --- |

## 2.4  Layer 3b — EnKF State Estimation

## The Ensemble Kalman Filter (EnKF) estimates the hidden internal states of the digester — specifically S2 (volatile fatty acids) and X2 (methanogen biomass) — from observable measurements alone: biogas flow rate, CH₄ percentage, and digester temperature.

## At farm scale, S2 and X2 are never directly measured. The EnKF uses the AD4 ODE as a physical process model, propagates an ensemble of 100 possible digester states forward in time, and updates them each day using the Kalman gain when new measurements arrive. Every estimate includes a 95% confidence interval and a probability of souring or washout.

| Tool | Purpose | Key output |
| --- | --- | --- |
| enkf_initialise | Configure filter with digester volume, HRT, influent COD | Confirmation of filter setup |
| enkf_update | Advance filter one day; reads plant_state automatically | S2_mean ± S2_std, souring_probability, risk_level, guidance |
| enkf_status | Current risk summary and 7-day S2 trend for morning briefing | operator_summary, days_tracked, S2_trend_7d |

## If the farm performs weekly FOS/TAC titration, the result can be passed as fos_mg_per_l to enkf_update(). This dramatically tightens the S2 estimate by providing a direct (approximate) VFA observation. Without FOS/TAC data, the filter still runs but S2 uncertainty remains wider.

| HONEST LIMIT | EnKF outputs are model estimates, not direct measurements. Always report S2 as “estimated S2” alongside the uncertainty interval. If S2_std > 20 mmol/L, the estimate is too uncertain to act on — request a FOS/TAC measurement before making operational decisions. Kinetic parameters remain unvalidated Benyahia defaults. |
| --- | --- |

3.  SCADA Integration and StateBuffer

## 3.1  Sensor Requirements

## The system degrades gracefully with fewer sensors. The following table reflects guidance from practical deployment experience:

| Sensor | Status | Notes |
| --- | --- | --- |
| biogas_flow_nm3h | REQUIRED | Minimum requirement to run AD4 with EnKF. Primary observation input to the Kalman filter. |
| ch4_pct | REQUIRED | Minimum requirement. Second observation input to the Kalman filter. Together with biogas_flow_nm3h, enables S2 and X2 estimation. |
| digester_temp_c | RECOMMENDED | Strongly recommended. Drives Arrhenius temperature correction in all AD4 and EnKF tools. A 3°C drop meaningfully reduces the safe operating envelope. |
| vfa_mmol_l (S2 proxy) | OPTIONAL | Makes state estimation significantly more reliable when available. The SENTRY Biogas Production Monitor is one relatively inexpensive option. Can also be supplied via weekly FOS/TAC titration passed to enkf_update(). |
| digester_ph, h2s_ppm, alkalinity | OPTIONAL | Enables VFA/alkalinity ratio calculation, H₂S alert thresholds, and fuller process diagnostics. |

## 3.2  SCADA Data Pipeline

## Raw SCADA exports (CSV or Excel) enter the system through a three-stage pipeline:

| Stage | Component | What happens |
| --- | --- | --- |
| 1 | scada_mapper.py | Auto-detects SCADA vendor from column patterns. Uses fuzzy regex matching to map raw column names (e.g. “TIC_101”, “Digester Temp (C)”) to internal variable names (digester_temp_c). Supports Siemens, Rockwell, Mitsubishi, and generic CSV formats. |
| 2 | StateBuffer.insert_live_data() | Each mapped row passes through CUSUM anomaly detection. Spikes are discarded (value set to NULL); clean values are written to plant_state.sqlite. Raw payloads are preserved in raw_telemetry for audit. |
| 3 | MCP get_plant_state() | The AI reads the latest clean row from StateBuffer. Forward-fill is applied so the EnKF always has a complete state vector even when some sensors report infrequently. |

## The AI can trigger this pipeline directly via the ingest_scada_file() tool when an operator provides a data file path. The vendor is auto-detected; no manual column mapping configuration is required.

## 3.3  CUSUM Anomaly Detection

## StateBuffer applies a CUSUM (Cumulative Sum) control chart to each monitored sensor variable. The algorithm maintains a rolling mean and two accumulators (S+ and S−). When either accumulator exceeds the decision threshold H, the reading is discarded as a spike and the accumulators are reset. Parameters are tunable per site via site_config.json.

| Variable | K (slack) | H (threshold) | Interpretation |
| --- | --- | --- | --- |
| digester_temp_c | 0.5 °C | 2.0 °C | Rejects readings deviating > ±2°C from rolling mean |
| digester_ph | 0.1 | 0.5 | Rejects pH spikes exceeding ±0.5 units |
| biogas_flow_nm3h | 10 Nm³/h | 50 Nm³/h | Allows for operational ramp-up before flagging anomalies |
| ch4_pct | 1.0 % | 5.0 % | Rejects sudden CH₄ composition jumps |
| h2s_ppm | 20 ppm | 100 ppm | Rejects H₂S sensor spikes while passing genuine exceedances |

4.  Calibration and Parameter Fitting

The system includes a structured calibration layer (biomethane_calibration.py) with honest reporting of what has and has not been fitted to data.

Calibration is manual. The operator edits parameters directly in biomethane_calibration.py — Buswell molecular formulas, C/N ratio tables, k6 fitting bounds — and re-runs bio_cli.py calibrate ... to apply changes. There is no AI-driven or automated parameter tuning; every calibration is an explicit, operator-initiated action.

| Parameter | Source | Status | Notes |
| --- | --- | --- | --- |
| D (dilution rate) | HRT column in dataset | FROM DATASET | Direct measurement |
| k6 (CH₄ yield) | Fitted to Q_CH₄ observations | FITTED TO DATA | Analytically solved; Buswell constrains bounds |
| Ki2 (Haldane inhibition) | Benyahia et al. (2012) | LITERATURE | S2 not measured at farm scale |
| mu2_max | Benyahia et al. (2012) | LITERATURE | T-corrected via Arrhenius (θ = 1.035) |
| Ks2, k2, k3 | Benyahia et al. (2012) | LITERATURE | Not identifiable without direct S2 measurements |

The generate_model_card.py script regenerates the RAG model card automatically after each calibration run, updating threshold tables and operating limits to reflect the current parameter set. A calibration_log.jsonl file maintains a timestamped history of every parameter set tried.

5.  Plant State Data Managed

The application maintains a live plant_state record in StateBuffer, updated from SCADA CSV files or manual entry:

| Variable | Description | Alert threshold |
| --- | --- | --- |
| digester_temp_c | Digester temperature (°C) | < 32°C or > 42°C (mesophilic) |
| digester_ph | pH of digester contents | < 6.8 or > 7.8 |
| vfa_mmol_l | Volatile fatty acids (S2 proxy, mmol/L) | > 8 mmol/L warning; > 30 critical |
| biogas_flow_nm3h | Biogas flow rate (Nm³/h) | EnKF observation input |
| ch4_pct | Methane percentage in biogas | EnKF observation input |
| h2s_ppm | Hydrogen sulphide concentration (ppm) | > 200 ppm warning; > 500 critical |
| biomethane_purity_pct | Upgraded gas purity (%) | < 97% (EN 16723-1 grid injection) |
| organic_load_kg_vs_d | Daily organic loading rate (kg VS/m³/day) | < 1.5 or > 3.5 kg VS/m³/day (VDI 4630) |
| hydraulic_retention_days | HRT — used to compute dilution rate D | < 15 days warning |
| fos_mg_per_l | Manual FOS/TAC titration result (mg CaCO₃/L). Sparse; lab results only, not CUSUM-filtered. | Passed to enkf_update() to tighten S2 estimate |

6.  Practical Value for Farm Operators

## 6.1  What the system does well

- Threshold interpretation — converts raw sensor numbers into plain-language status (HEALTHY / WATCH / WARNING / CRITICAL) with specific corrective actions

- Feedstock planning — Buswell BMP and C/N ratio calculations before changing feedstock mix; blend_feedstocks() evaluates multi-stream recipes for OLR, C/N, and inhibition risk

- Seasonal safety margin — temperature-corrected D_crit tells the operator how much their safe operating envelope has shrunk in winter

- Daily VFA trend monitoring — EnKF S2_trend_7d provides an early warning of rising VFA before it reaches dangerous levels

- Acidification monitoring — get_vfa_alkalinity_ratio() gives the FOS/TAC ratio directly, with a plain-language status and advice

- What-if scenario testing — perturbation simulator answers “what happens if I get a high-COD delivery for 7 days” before the delivery arrives

- Privacy — all computation runs locally; no farm data transmitted externally

## 6.2  Honest limitations

- Quantitative methane forecasting — the AD4 simulator is not calibrated for production prediction; use Buswell + OLR calculations for yield estimates

- pH and alkalinity — the 4-state model has no alkalinity state; FOS/TAC ratio and bicarbonate buffering are handled by alert thresholds and the VFA/alkalinity tool, not by the physical model

- Ammonia inhibition — not represented in the 4-state model; high TAN (> 3000 mg/L) scenarios trigger alerts but are not simulated

- Time-series historian — the StateBuffer retains a 48-hour rolling window of clean data; longer-term trend queries require replay_dataset.py

- Local model limitations — the local language model handles lookup-style questions well but may struggle with multi-factor reasoning across several tool outputs simultaneously; complex diagnoses may require multiple sequential questions

7.  Example Scenario

An operator notices biogas flow has dropped 18% over three days.

| Step | What happens |
| --- | --- |
| 1 | Operator asks: “My biogas dropped 18% — what should I check?” |
| 2 | AI calls enkf_status() — reads estimated S2_mean = 47 mmol/L (WATCH), S2_trend_7d = RISING, souring_probability = 0.22 |
| 3 | AI calls ad4_critical_dilution_rate() — reads digester_temp_c = 31°C from StateBuffer, reports D_crit = 0.451 at this temperature vs 0.533 at 35°C, safety margin 89% (SAFE) |
| 4 | AI calls get_vfa_alkalinity_ratio() — FOS/TAC ratio = 0.38 (HIGH), advice: “Ratio elevated — monitor closely for acidification risk” |
| 5 | AI responds: “Your estimated VFA is 47 mmol/L and rising over 7 days — early warning of overloading, not yet critical. Your FOS/TAC ratio (0.38) is elevated. Washout margin is safe at current temperature. Recommend: reduce feeding rate by 15–20% for 5 days and monitor VFA trend. Request a FOS/TAC measurement to confirm the estimate.” |

8.  Technical References and Standards

## Standards

- VDI 4630:2016 — Fermentation of organic materials (temperature, OLR thresholds)

- EN 16723-1:2016 — Biomethane quality for grid injection (> 97% CH₄ purity)

## Key Literature

- Bernard et al. (2001). Dynamical model development and parameter identification for an anaerobic wastewater treatment process. Biotechnology and Bioengineering, 75(4), 424–438. — Source of AM2 ODE structure and kinetic expressions.

- Benyahia et al. (2012). Predictive uncertainty quantification of a model of anaerobic digestion. — Source of default kinetic parameters (Ki2, mu2_max, k1–k6).

- Buswell & Mueller (1952). Mechanism of methane fermentation. Ind. Eng. Chem., 44(3), 550–552. — Theoretical BMP equation used in Layer 2 tools.

- Evensen, G. (1994). Sequential data assimilation with a nonlinear quasi-geostrophic model. J. Geophys. Res., 99(C5). — EnKF algorithm foundation.

- Rittmann & McCarty (2001). Environmental Biotechnology: Principles and Applications. — Source of Arrhenius theta = 1.035 for mesophilic methanogen temperature correction.

- Page, E.S. (1954). Continuous inspection schemes. Biometrika, 41(1/2), 100–115. — CUSUM outlier detection.

- Weiland, P. (2010). Biogas production: current state and perspectives. Applied Microbiology and Biotechnology, 85(4). — OLR ranges (1.5–3.5 kg VS/m³/day).

- Benyahia, B., et al. (2012). "Anaerobic Digestion Model No. 1 (ADM1).”

- Rittmann, B.E. & McCarty, P.L. (2001). Environmental Biotechnology.

## Test datasets Used

- Indian Biogas Production Dataset (Kaggle, 2010–2024, 15,298 rows) — feedstock composition, biogas yield, digester temperature, C/N ratio. Used for k6 fitting via analytical single-parameter calibration.

- Farm-scale Biodigester Performance Dataset (Mendeley Data, ri_flex.csv) — daily measurements of temperature, pH, VFA, TAN, biogas flow, CH₄%, CO₂%. Used for plant_state calibration and CUSUM outlier filtering.

9.  Understanding the Technology: MCP, AI, and RAG

## 9.1  The AI Client and MCP Server

## To understand how this system works, it helps to think about two distinct roles: the AI Client (the “brain”) and the MCP Server (the “specialist toolbox”).

## General-purpose AI models are highly capable at language — explaining concepts, interpreting numbers, and answering questions in plain English. But on their own, they cannot see your plant’s sensors in real time, nor can they perform the specific physics calculations your digester requires. They also have no knowledge of your particular feedstock mix, digester volume, or operating history.

## MCP stands for Model Context Protocol. It is a standardised bridge that lets an AI call specific, trusted tools. This script acts as the MCP Server: it provides the AI with a structured toolbox containing sensor reads, safety rules, thermodynamic equations, and a calibrated physical simulator. When you ask “Is my digester safe right now?”, the AI crosses the MCP bridge to read your sensors, run the appropriate calculations, and explain the results in plain language. The AI does the talking; the MCP server does the maths.

## 9.2  What is RAG?

## RAG stands for Retrieval-Augmented Generation. It is the mechanism that gives the AI access to a curated library of domain knowledge — without needing to train a specialist model from scratch.

## Here is how it works in plain terms. Imagine you have a shelf of technical manuals: the AD4 model card, the Bernard (2001) equations, the API reference, and the failure-mode lookup tables. Each page of each manual has been converted into a numerical fingerprint (called an embedding) and stored in a database (ChromaDB). When you ask a question, the system looks up which pages have fingerprints closest to your question, retrieves those pages, and hands them to the AI along with your question. The AI then answers based on both its general language knowledge and the specific retrieved pages.

## The practical effect is that a small, locally running model — one that fits comfortably on a Mac mini and never sends data off the farm — can answer precise questions about FOS/TAC thresholds, Haldane inhibition kinetics, or EN 16723-1 grid injection standards, because the relevant content is retrieved and placed directly in front of it at query time. The model does not need to have memorised every technical detail; it just needs to read and reason.

## For this system, the RAG documents are written in a deliberately dense, lookup-style format — one equation per chunk, one failure mode per table row — because small models retrieve and use short, explicit passages more reliably than long narrative text. The generate_model_card.py script regenerates the AD4 model card automatically after each calibration run, so the retrieved knowledge always reflects the current parameter set.

| NOTE | RAG is not perfect. If the right document chunk is not retrieved, the AI will not have the right information. This is why the RAG library is kept small and purpose-written rather than large and general. Fewer, more precise documents outperform a large generic corpus for a 4B parameter model. |
| --- | --- |

Appendix list of useful terminology

## VFA = Volatile Fatty Acids (mainly acetate, propionate, butyrate)

## FOS = Fermentable Organic Acids (same as VFA)

## TAC = Total Alkalinity Capacity (buffering capacity)

## HRT = Hydraulic Retention Time (days), Time the substrate spends in the digester before being washed out.

Appendix  AD4 4-state simulator

State Variables (AD4State)

| Symbol | Name | Units | Description |
| --- | --- | --- | --- |
| S1 | Substrate | g COD/L | Soluble chemical oxygen demand in the digester |
| S2 | VFA | mmol/L | Volatile fatty acids - key health indicator |
| X1 | Acidogens | g VSS/L | Acidogenic biomass (fermenters) that convert S1→S2 |
| X2 | Methanogens | g VSS/L | Methanogenic biomass that convert S2→CH4 |

## Health Thresholds

| State | S2 (mmol/L) | S2 (mg/L) | Risk |
| --- | --- | --- | --- |
| HEALTHY | < 10 | < 600 | Low - normal operation |
| WARNING | 10-30 | 600-1800 | Monitor - possible overload |
| CRITICAL | > 30 | > 1800 | Souring imminent - take action |

Kinetic Parameters (AD4Params)

## Acidogens (X1 / S1 → S2)

| Symbol | Name | Default | Units | Description |
| --- | --- | --- | --- | --- |
| mu1_max | Max growth rate | 1.20 | d⁻¹ | Maximum specific growth rate |
| Ks1 | Half-saturation | 7.10 | g COD/L | Substrate concentration at half-max growth |
| k1 | COD yield | 42.14 | - | g COD consumed per g COD input |
| k2 | VFA yield | 116.5 | mmol/g | VFA produced per g COD consumed |

## Methanogens (X2 / S2 → CH4)

| Symbol | Name | Default | Units | Description |
| --- | --- | --- | --- | --- |
| mu2_max | Max growth rate | 0.74 | d⁻¹ | Maximum specific growth rate |
| Ks2 | Half-saturation | 9.28 | mmol/L | VFA concentration at half-max growth |
| Ki2 | Inhibition constant | 256 | mmol/L | Haldane inhibition - high S2 slows growth |
| k3 | VFA consumption | 268 | mmol/g | VFA consumed per g methanogen biomass |

## Gas Production & Biomass

| Symbol | Name | Default | Units | Description |
| --- | --- | --- | --- | --- |
| k6 | CH4 coefficient | 453 | mL CH4/mmol S2 | CH4 produced per mmol VFA consumed |
| alpha | Washout factor | 1.0 | - | 1.0 = CSTR (no retention), <1 = settler/retention |

Temperature Correction

| Symbol | Name | Default | Description |
| --- | --- | --- | --- |
| theta | Arrhenius coefficient | 1.035 | Growth rate multiplier per °C deviation |
| T_ref | Reference temperature | 35.0 | °C at which kinetics were calibrated |

### Temperature Correction Formula

mu2_max(T) = mu2_max × theta^(T - T_ref)

Example: At 32°C (vs 35°C reference):

- mu2_max = 0.74 × 1.035^(-3) = 0.667 d⁻¹

Derived Quantities

| Symbol | Name | Units | Formula |
| --- | --- | --- | --- |
| Q_CH4 | Methane production | mL/L/d | k6 × μ2(S2) × X2 |
| D | Dilution rate | d⁻¹ | 1 / HRT |
| OLR | Organic loading rate | g COD/L/d | S1_in × D |

Model Equations

## Growth Kinetics

### Acidogens (Monod):

μ1 = μ1_max × S1 / (Ks1 + S1)

### Methanogens (Haldane):

μ2 = μ2_max × S2 / (Ks2 + S2 + S2²/Ki2)

## Mass Balance (CSTR)

dS1/dt = D(S1_in - S1) - k1·μ1·X1

dS2/dt = D(0 - S2) + k2·μ1·X1 - k3·μ2·X2

dX1/dt = μ1·X1 - D·X1/α

dX2/dt = μ2·X2 - D·X2/α

## Methane Production

Q_CH4 = k6 × μ2(S2) × X2  [mL/L/d]

Model Limitations

- Assumes perfect mixing (CSTR)

- k6 is the only calibrated parameter - other kinetics use Benyahia defaults (parameters in Anaerobic Digestion Model No.1 (ADM1) by BenvahiaB. Et al. (2012)).

- Temperature range: 20-40°C (Arrhenius extrapolation outside this Usage in MCP Tools

## AD4 Simulation

ad4_simulate(dilution_rate=0.05, influent_cod_g_per_l=25.0, days=100)

## EnKF State Estimation

enkf_initialise(digester_volume_m3=2000, hrt_days=20, s1_in_g_per_l=25.0)

enkf_update()  # Uses biogas_flow_nm3h from plant_state

## Calibration (k6 only)

ad4_fit_k6(measured_ch4_ml_per_l_d=100)

Sensor Requirements Analysis

The absolute minimum sensor variables needed to run the AD4 simulator with EnKF are:

## Required (2 variables):

- biogas_flow_nm3h - biogas flow rate

- ch4_pct - methane percentage

These calculate Q_CH4 (methane flow), the primary observation for EnKF. The EnKF then estimates missing/hidden states (S2, X2, S1, X1) from these observations. is uncertain)

- Cannot measure S2 directly - use EnKF for stochastic estimation from Q_CH4

## Optional but recommended:

- digester_temp_c - temperature (defaults to 35.0°C if missing)

- fos_mg_per_l - FOS/TAC weekly measurement (improves S2 estimate)

## Fixed parameters (not sensor variables, but required):

- digester_volume_m3 - for Q_CH4 conversion

- hydraulic_retention_days - for dilution rate D

- S1_in - influent COD (default 25.0 g/L)

## Values of having a direct S2 sensor:

Having a direct S2 (VFA) sensor would be a meaningful improvement over the current setup, though the gains are modest.

What genuinely improves:

The EnKF would correct S2 directly rather than inferring it from biogas flow and CH₄%, which currently lags actual VFA accumulation by hours to days. Weekly FOS/TAC titration already provides this correction sparsely — a continuous sensor makes it routine.

What remains difficult:

Calibrating Ki2 and Ks2 from operational data alone is difficult. These parameters are structurally correlated in the AD4 model — multiple combinations can fit the same observations equally well. Resolving them properly requires carefully designed perturbation experiments, not just routine sensor data. Literature defaults may remain the practical choice even with S2 available.

Spatial variability is also a real concern. A farm-scale digester is not a well-stirred lab reactor. A single probe reading may not represent the bulk VFA concentration, introducing a systematic bias that no amount of filtering corrects.

The binding constraint:

The AD4 model is a 4-state simplification of a vastly more complex microbial ecosystem. Better sensors improve the state estimation layer. They do not make the model more accurate in a fundamental sense. At farm scale, the model structure itself — not the data — is ultimately the limiting factor.

A continuous S2 sensor is worth having. But expectations should be calibrated accordingly.